UMES Environmental Health and Safety

Maintenance Shop Safety Checklist

2026 Edition — Updated for Current OSHA Standards

How this checklist is organized

Part 1 contains the Universal Program Checklist — requirements that apply to every maintenance shop regardless of trade. Part 2 contains a supplemental, shop-specific checklist for each of the ten shops listed below, covering the hazards unique to that trade.

On a monthly basis Use Part 1 plus the relevant shop section from Part 2 for each inspection.

Shops covered:

1. Motorpool

2. Paint Shop

3. Electrical Shop

4. HVAC Shop

5. Plumbing Shop

6. Locksmith Shop

7. Material Handling Shop

8. Housekeeping Shop

9. Carpentry Shop

10. Grounds Shop

# Part 1 — Universal Program Checklist

CHEMICAL SAFETY

| ☐ | Hazardous waste accumulation containers are in good condition, closed when not actively being used, and stored at point of generation with proper segregation. |
| --- | --- |
| ☐ | Chemicals are stored according to compatibility (segregated by hazard class, not alphabetically). |
| ☐ | A chemical spill kit appropriate to the shop's hazards is immediately available and fully stocked. |
| ☐ | Corrosive chemicals are stored below eye level |
| ☐ | Chemical storage areas are free of consumables; nothing is stored on workbenches, near sinks, or near drains. |

ENVIRONMENTAL AND SUSTAINABILITY

| ☐ | Spill prevention and secondary containment measures are maintained where oils, fuels, or chemicals are stored |
| --- | --- |
| ☐ | Universal and Electrical Waste is properly accumulated, labeled, and managed |
| ☐ | Waste materials are properly segregated to support recycling, reuse, and waste minimization where applicable |
| ☐ | Waste streams are managed to prevent releases to storm drains or the environment |
| ☐ | Regulated wastes are stored and managed in accordance with applicable requirements |

HAZARD COMMUNICATION (HAZCOM / GHS)  —  UPDATED FOR 2026

| ☐ | SDS are accessible to all shift workers (physical binder or verified electronic access). |
| --- | --- |
| ☐ | Hazardous waste containers are labeled “Hazardous Waste”, dated, and segregated by compatibility class. |
| ☐ | Hazard warning signs are installed, current, and in accordance with GHS symbols |
| ☐ | Chemicals are dated when received and again when opened. |
| ☐ | Chemical containers are intact, labeled, and closed with tight-fitting lids. |
| ☐ | Secondary chemical containers are properly labeled with product name and hazard warnings, per the updated GHS pictogram set. |
| ☐ | Compressed gas cylinders are positioned so labels are visible, restrained, and stored in ventilated area protected from heat sources. |

EMERGENCY PREPAREDNESS, FIRE, AND LIFE SAFETY

| ☐ | Emergency contact numbers are posted and current. |
| --- | --- |
| ☐ | Employees know how to report emergencies, injuries, spills, incidents, near-misses, and safety concerns. As well are informed they can report anonymously |
| ☐ | Emergency equipment and emergency shutoffs are clearly identified and readily accessible. |
| ☐ | No smoking in unauthorized areas. |
| ☐ | Exits, aisles, fire extinguishers, fire-alarm pull stations, alarm audio-visuals, sprinklers, and electrical panels remain clear and unobstructed. |
| ☐ | There is sufficient lighting of evacuation routes; emergency lighting and exit signs operate properly and readily visible |
| ☐ | 18” of vertical clearance is maintained below fire-sprinkler heads, including over shelving. |
| ☐ | No unauthorized modifications have been made to the facility structure (no user-built walls/partitions). |
| ☐ | No storage is kept on top of flammable-liquid storage cabinets. |
| ☐ | Any refrigerator/freezer used for flammable or combustible materials is rated appropriately for that use. |
| ☐ | No more than 1 liter of flammables is stored in refrigerators/freezers. |
| ☐ | Combustible materials are stored away from heat sources. |
| ☐ | Combustible waste and garbage are contained and removed on a regular schedule. |
| ☐ | First aid kits are available, stocked, and inspected on a routine schedule. |
| ☐ | No more than 10 gallons of flammable liquids are stored outside approved flammable-liquid storage cabinets |
| ☐ | Emergency shower/eyewash is accessible within 10 seconds, functional, and documentation of weekly activation is current. |

HEAT ILLNESS PREVENTION  —  UPDATED FOR 2026

| ☐ | A written heat illness prevention plan is in place for staff who work outdoors or in hot indoor environments. |
| --- | --- |
| ☐ | Cool drinking water, shade or a cooled rest area, and paid rest breaks are available during high-heat work. |
| ☐ | An acclimatization schedule is used for new employees or those returning after time away from hot-environment work. |
| ☐ | Supervisors and workers are trained to recognize the signs of heat exhaustion/heat stroke and know the emergency response steps. |
| ☐ | Heat index or WBGT readings are used to trigger additional precautions (extra breaks, adjusted workload) during hot conditions. |

INDUSTRIAL HYGIENE

| ☐ | No eating, drinking, or cosmetic applications occur in the shop. |
| --- | --- |
| ☐ | At least one sink is equipped with soap and paper towels. |
| ☐ | An appropriate disinfectant/decontamination solution is available |
| ☐ | Hepatitis B vaccination has been offered to employees exposed to bloodborne pathogens |
| ☐ | Respiratory hazards have been evaluated and appropriate respiratory protection identified |
| ☐ | Hearing hazards have been evaluated and appropriate hearing conservation efforts identified |
| ☐ | Exposure assessments have been completed where required |
| ☐ | Indoor Air Quality concerns have been reported to EHS |
| ☐ | Potential contaminated surfaces have been reported to EHS |
| ☐ | Potential moisture damage (discoloration, deterioration) have been reported to EHS |
| ☐ | Moisture sources (leaks, condensation, infiltration, etc) have been reported to EHS |
| ☐ | Asbestos-related concerns have been reported to EHS |
| ☐ | Lead-related concerns have been reported to EHS |

OCCUPATIONAL HEALTH AND SAFETY

| ☐ | Cabinets, furniture, and equipment taller than 4 feet are braced or anchored. |
| --- | --- |
| ☐ | There is no overhead storage of heavy materials. |
| ☐ | Walking surfaces are free of slip, trip, and fall hazards. |
| ☐ | Work surfaces are reasonably organized and free of clutter. |
| ☐ | Hand and power tools are inspected on a routine schedule; damaged tools are tagged out and removed from service. |
| ☐ | Portable ladders are in good condition, inspected before use, and used appropriately |
| ☐ | Manual material handling tasks and workstations are arranged to minimize ergonomic risk and musculoskeletal injuries |

PERSONAL PROTECTIVE EQUIPMENT  —  UPDATED FOR 2026

| ☐ | Required PPE is available, maintained, and appropriate for the hazards identified in the hazard assessment. |
| --- | --- |
| ☐ | Workers using respirators have been medically cleared, fit-tested and demonstrate competency in usage. |
| ☐ | Hazard Assessments were conducted in identifying appropriate PPE, completed assessments are maintained and available upon request |

EQUIPMENT AND ELECTRICAL SAFETY  —  UPDATED FOR 2026

| ☐ | Equipment is in serviceable condition and inspected on a routine schedule. |
| --- | --- |
| ☐ | Equipment is powered with permanent wiring or a fused electric strip (no 3-way plugs). |
| ☐ | Multi-outlet strip cords are 6' or less and have a built-in circuit breaker. |
| ☐ | No exposed “live” connections exist on electrical equipment. |
| ☐ | There is at least 36” of clearance around electrical panels. |
| ☐ | Extension cords do not run through walls, ceilings, or doors. |
| ☐ | GFCI protection is present on outlets near water. |
| ☐ | Machine parts are guarded (belts, pulleys, and other exposed moving parts). |
| ☐ | Electrical panel arc-flash labels are current and legibly show nominal voltage and the date of the arc-flash assessment. |
| ☐ | Equipment requiring lockout/tagout has an established energy isolation procedure available to authorized employees, devices in good condition, and used during servicing and maintenance activities. |

# Part 2 — Shop-Specific Supplemental Checklists

Each shop below adds trade-specific items to the Part 1 Universal Program Checklist.

# 1. Motorpool — Supplemental Safety Checklist

In addition to the universal program requirements, Motorpool areas that lift, fuel, or service vehicles must also meet the following.

| ☐ | Vehicle lifts are load-rated, inspected at least annually, and the rated capacity is posted and legible. |
| --- | --- |
| ☐ | Wheel chocks or jack stands are used any time a vehicle is raised or a wheel is removed. |
| ☐ | Battery charging areas are ventilated, and eye protection plus an apron/gloves are used when handling batteries or electrolyte. |
| ☐ | Used oil, antifreeze, and other fluids are stored in labeled, compatible containers with secondary containment. |
| ☐ | Compressed air tools are guarded and line pressure is regulated to a safe working level. |
| ☐ | Service pits and lift areas have appropriate guarding or fall protection. |
| ☐ | Fuel storage/transfer areas are separated from ignition sources, and containers are bonded/grounded during transfer. |
| ☐ | Vehicle exhaust removal (tailpipe extraction or ventilation) is functional wherever engines are run indoors. |
| ☐ | Multi-piece rim/wheel servicing uses an approved restraint device. |

# 2. Paint Shop — Supplemental Safety Checklist

In addition to the universal program requirements, spray-finishing and coatings work must also meet the following.

| ☐ | Spray booth ventilation is functioning and inspected/certified per NFPA 33. |
| --- | --- |
| ☐ | Respiratory protection (organic-vapor and particulate, as appropriate) is required, available, and fit-tested for spray operations. |
| ☐ | Flammable and combustible paints/solvents are stored in approved flammable-liquid cabinets; containers are bonded and grounded during transfer. |
| ☐ | Electrical fixtures within the spray booth and spray area are explosion-proof/rated for the classified location. |
| ☐ | No open flame or unprotected ignition source is present near the spray booth or mixing area. |
| ☐ | Products containing methylene chloride or isocyanates are identified, and applicable exposure monitoring/medical surveillance is in place. |
| ☐ | Spray booth filters are changed on schedule, and spent filters are stored/disposed of per fire code. |
| ☐ | Chemical-resistant gloves and splash goggles are worn when handling paints, thinners, and strippers. |

# 3. Electrical Shop — Supplemental Safety Checklist

In addition to the universal program requirements, work on electrical systems and equipment must also meet the following.

| ☐ | Qualified vs. unqualified worker roles are documented; only qualified persons perform work on or near energized equipment. |
| --- | --- |
| ☐ | An arc-flash risk assessment has been completed for panels/equipment worked on, and labels show incident energy/PPE category, nominal voltage, and the assessment date. |
| ☐ | Arc-rated PPE (clothing, face shields/hoods) is available, in good condition, and used for energized work. |
| ☐ | Individually assigned LOTO locks and tags are used, documented, and made available for inspection upon request |
| ☐ | Insulated hand tools are rated for the voltage present and are visually inspected before each use. |
| ☐ | “Test before touch” is practiced: voltage testers are verified on a known live source immediately before and after use. |
| ☐ | Panels and boards have current, accurate circuit directories. |
| ☐ | GFCI protection is tested on a routine schedule. |

# 4. HVAC Shop — Supplemental Safety Checklist

In addition to the universal program requirements, refrigerant handling and rooftop/mechanical-room work must also meet the following.

| ☐ | Refrigerant is handled only by EPA Section 608–certified technicians. |
| --- | --- |
| ☐ | Systems containing 1,500 lbs or more of a high-global-warming-potential refrigerant have an automatic leak detection system installed. |
| ☐ | Refrigerant recovery equipment is available and used; refrigerant is never intentionally vented to atmosphere. |
| ☐ | Fall protection (guardrails, anchor points, or warning lines) is used for rooftop unit access and servicing. |
| ☐ | LOTO is applied before servicing fans, compressors, or other equipment that could start unexpectedly. |
| ☐ | Ladders and lifts used for rooftop or elevated access are inspected before each use. |
| ☐ | Hot work permits are filled out and approved before any hot work is performed, completed permits are made available upon request. Fire watch present |

# 5. Plumbing Shop — Supplemental Safety Checklist

In addition to the universal program requirements, plumbing  and wastewater maintenance  must also meet the following.

| ☐ | Hot work permits are filled out and approved before any hot work is performed, completed permits are made available upon request. Fire watch present. |
| --- | --- |
| ☐ | Silica exposure controls (wet cutting or dust extraction) are used when cutting pipe, concrete, or masonry. |
| ☐ | Waterproof gloves, boots, and eye protection are worn for wastewater-exposure tasks. |

# 6. Locksmith Shop — Supplemental Safety Checklist

In addition to the universal program requirements, bench and installation work must also meet the following.

| ☐ | Eye protection is worn when drilling, grinding, or machining locks, keys, or hardware. |
| --- | --- |
| ☐ | Bench workstations are set up ergonomically for repetitive keying and fitting tasks. |
| ☐ | Task lighting at workbenches is adequate for close-detail work. |
| ☐ | Ladders used for door and access-hardware installation are inspected and rated for the task. |

# 7. Material Handling Shop — Supplemental Safety Checklist

In addition to the universal program requirements, powered industrial truck and warehousing operations must also meet the following.

| ☐ | Powered industrial truck (forklift) operators are certified per 29 CFR 1910.178, with refresher training every three years or after a relevant incident/near-miss. |
| --- | --- |
| ☐ | A documented pre-operation inspection is completed for each powered industrial truck before each shift. |
| ☐ | Load capacity plates are legible, and loads are kept within rated capacity. |
| ☐ | Rigging and lifting equipment (slings, chains, hoists) is inspected before use and marked with rated capacity. |
| ☐ | Dock plates/levelers are secured, and trailers are restrained (wheel chocks or ICC bars) during loading/unloading. |
| ☐ | Pedestrian and powered-truck traffic are separated with marked aisles and convex mirrors at blind corners. |
| ☐ | Storage racking is inspected, and posted load limits are followed. |
| ☐ | Palletized/stacked materials are stable and do not exceed safe storage height limits. |

# 8. Housekeeping Shop — Supplemental Safety Checklist

In addition to the universal program requirements, custodial and cleaning operations must also meet the following.

| ☐ | incompatible chemicals (e.g., bleach and ammonia-based products) are never mixed. |
| --- | --- |
| ☐ | Wet-floor signage is displayed during and immediately after mopping or floor cleaning. |
| ☐ | Vacuum and equipment cords are inspected and routed to avoid trip hazards. |
| ☐ | Gloves and eye protection are worn for chemical handling and biohazard cleanup. |

# 9. Carpentry Shop — Supplemental Safety Checklist

In addition to the universal program requirements, woodworking and stationary machinery must also meet the following.

| ☐ | Stationary machinery (table saws, jointers, planers) is guarded per 29 CFR 1910.213. |
| --- | --- |
| ☐ | Blade guards and riving knives/splitters are in place and functional on all saws. |
| ☐ | A dust collection system is operating, and combustible wood-dust accumulation is controlled on ledges, beams, and equipment. |
| ☐ | Hearing protection is available and required near loud machinery. |
| ☐ | Silica exposure controls are used when cutting composite, cement-board, or engineered materials. |
| ☐ | Push sticks/blocks are available and used to keep hands clear of blades. |
| ☐ | Compressed air for pneumatic nailers is regulated, and workers have received nail-gun-specific safety training. |

# 10. Grounds Shop — Supplemental Safety Checklist

In addition to the universal program requirements, outdoor crews and grounds equipment must also meet the following.

| ☐ | The heat illness prevention plan is actively followed |
| --- | --- |
| ☐ | Pesticide/herbicide applicators are certified as required |
| ☐ | Mower and trimmer blade guards are in place, and eye, hearing, and leg protection are worn during operation. |
| ☐ | Gasoline and diesel are stored in approved safety cans, away from ignition sources. |
| ☐ | Chainsaw operators are trained, and chaps, helmet, and eye/hearing/hand protection are used. |
| ☐ | ATV/UTV operators are trained, and rollover protection and seatbelts are used. |
| ☐ | Hydration and shaded/cooled rest areas are provided and used during hot-weather work. |